The purpose of a MAC address table, often referred to as a CAM (Content Addressable Memory) table, is to enable a Layer 2 switch to efficiently and intelligently forward Ethernet frames. By dynamically mapping the unique hardware address of every connected device to a specific switch port, the switch avoids unnecessarily sending unicast traffic across the entire local network segment, which significantly reduces network congestion and optimizes bandwidth usage.

The MAC address table facilitates the core, ongoing functions of a Layer 2 switch:

- Learning (MAC Address Learning): The switch dynamically builds its table by examining the source MAC address of every incoming frame. It records this address and associates it with the port where the frame was received. The table also uses aging timers to remove entries if they are inactive for a set duration, ensuring the table stays current when devices move.

- Forwarding/Filtering (Frame Forwarding Decisions): When a new frame arrives, the switch inspects the destination MAC address in the frame's header.

  - If the destination MAC address is found in the table, the switch forwards the frame only out the corresponding port (Forwarding).

  - If the destination MAC address is mapped to the same port the frame was received on, the switch drops the frame (Filtering).

- Flooding (Handling Unknown Traffic): If the destination MAC address is not found in the table (an unknown unicast frame) or is the universal broadcast address (FF:FF:FF:FF:FF:FF), the switch forwards the frame out all ports except the port on which it was received.