What Does "copy running-config startup-config" Do—And When Should You Use It?

On Cisco routers and switches, the command copy running-config startup-config is used to save your device’s configuration.

What Does It Do?

- When you make changes to a device (like adding IP addresses, setting passwords, or configuring interfaces), those changes are stored in RAM. This is called the running configuration.

- RAM is temporary. If the device reboots or loses power, everything in RAM is lost.

- The command copies the running configuration from RAM into NVRAM, which is non-volatile memory. In NVRAM, it is saved as the startup configuration, which the device automatically loads the next time it powers on.

In other words, this command is like hitting Save on a document—you’re making sure your work won’t disappear.

Why Is It Important?

If you forget to use this command, any changes you made will vanish after a reboot. The device will fall back to the last saved startup configuration, which may not include your updates.

When Should You Use It?

You should use copy running-config startup-config:

- After every important change you want to keep – For example, if you configure a new VLAN or set up routing, run this command right away.

- Before rebooting or doing maintenance – Just like saving a file before shutting down your computer, this prevents data loss.

Real-World Example

Imagine you log into a switch and set a new password:

Switch> en

Switch# conf t

Switch(config)# enable secret class123

At this point, the new password exists only in the running configuration. If the switch restarts now, the password will be lost. To keep it, you must save it with:

Switch# copy running-config startup-config

Now, even if the switch powers off and back on, the password is still there—because it’s saved in the startup configuration.

Summary

The command copy running-config startup-config ensures your configuration changes are saved permanently. Always run it after making changes you want to keep, and especially before restarting a device. It’s a simple habit that can save you from losing hours of work.