- Difference Between Static and Dynamic Routing

| Feature | Static Routing | Dynamic Routing |
| --- | --- | --- |
| Definition | Routing table entries are manually configured by a network administrator. | Routing table entries are learned, maintained, and updated automatically by routers running a routing protocol (e.g., OSPF, EIGRP, RIP). |
| Updates | None. Routes only change if an administrator manually modifies or removes them. | Automatic. Routers exchange information with neighbors to automatically discover new networks, update best paths, and quickly converge after a topology change. |
| Complexity/Overhead | Low. Requires minimal CPU or bandwidth overhead on the router. | High. Requires more CPU and memory to run the protocol, and consumes bandwidth for control traffic (updates/hellos). |
| Security | High. Administrators have complete control; no unexpected route advertisements. | Lower (by default). Routers share topology information, potentially advertising networks to unauthorized devices. |
| Scalability | Poor. Not feasible for large, complex, or frequently changing networks. | Excellent. Required for large, multi-network enterprise or internet environments. |

Why Networks Cannot Have Just One or the Other

We cannot rely exclusively on a single method because the two mechanisms are designed to fulfill different, non-overlapping operational needs:

- Static Routing is Essential for Exits: Static routes are typically used for connecting to a specific next hop that will not change, most commonly configuring a default route (known as the Gateway of Last Resort). This single route tells the router where to send traffic for any network not explicitly found in its routing table (i.e., the path to the ISP or the rest of the internet).

- Dynamic Routing is Essential for Scale/Redundancy: Dynamic routing is required for internal networks because manually managing thousands of routes in a large enterprise is impossible. Furthermore, dynamic protocols provide automatic redundancy—if a main link fails, the protocol automatically calculates and switches to the next best path, a critical function static routing cannot do.

- Security and Predictability: In practice, organizations use a combination of the two:

  - Static Routes are used for simple paths, management links (e.g., pointing a router to a specific server), and as the secure gateway to the Internet.

  - Dynamic Routes are used to handle the complexity and changes within the core of the network.

The final network is therefore a hybrid, leveraging static routes for simplicity and security on the edge and dynamic routing for agility and scale in the core.