# Packet Tracer - Use Ping and Traceroute to Test Network Connectivity

# Addressing Table

| Device | Interface | IP Address / Prefix |  | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 2001:db8:1:1::1/64 |  | N/A |
| R1 | G0/1 | 10.10.1.97 | 255.255.255.224 | N/A |
| R1 | S0/0/1 | 10.10.1.6 | 255.255.255.252 | N/A |
| R1 | S0/0/1 | 2001:db8:1:2::2/64 |  | N/A |
| R1 | S0/0/1 | fe80::1 |  | N/A |
| R2 | S0/0/0 | 10.10.1.5 | 255.255.255.252 | N/A |
| R2 | S0/0/0 | 2001:db8:1:2::1/64 |  | N/A |
| R2 | S0/0/1 | 10.10.1.9 | 255.255.255.252 | N/A |
| R2 | S0/0/1 | 2001:db8:1:3::1/64 |  | N/A |
| R2 | S0/0/1 | fe80::2 |  | N/A |
| R3 | G0/0 | 2001:db8:1:4::1/64 |  | N/A |
| R3 | G0/1 | 10.10.1.17 | 255.255.255.240 | N/A |
| R3 | S0/0/1 | 10.10.1.10 | 255.255.255.252 | N/A |
| R3 | S0/0/1 | 2001:db8:1:3::2/64 |  | N/A |
| R3 | S0/0/1 | fe80::3 |  | N/A |
| PC1 | NIC | 10.10.1.98 | 255.255.255.224 | 10.10.1.97 |
| PC2 | NIC | 2001:db8:1:1::2/64 |  | fe80::1 |
| PC3 | NIC | 10.10.1.18 | 255.255.255.240 | 10.10.1.17 |
| PC4 | NIC | 2001:db8:1:4::2/64 |  | fe80::3 |

# Objectives

Part 1: Test and Restore IPv4 Connectivity

Part 2: Test and Restore IPv6 Connectivity

# Scenario

There are connectivity issues in this activity. In addition to gathering and documenting information about the network, you will locate the problems and implement acceptable solutions to restore connectivity.

Note: The user EXEC password is cisco. The privileged EXEC password is class.

# Instructions

## Test and Restore IPv4 Connectivity

### Use ipconfig and ping to verify connectivity.

Click PC1 and open the Command Prompt.

Enter the ipconfig /all command to collect the IPv4 information. Complete the Addressing Table with the IPv4 address, subnet mask, and default gateway.

Click PC3 and open the Command Prompt.

Enter the ipconfig /all command to collect the IPv4 information. Complete the Addressing Table with the IPv4 address, subnet mask, and default gateway.

Use the ping command to test connectivity between PC1 and PC3. The ping should fail.

### Locate the source of connectivity failure.

From PC1, enter the necessary command to trace the route to PC3.

#### Question:

What is the last successful IPv4 address that was reached?

10.10.1.97

The trace will eventually end after 30 attempts. Enter Ctrl+C to stop the trace before 30 attempts.

From PC3, enter the necessary command to trace the route to PC1.

#### Question:

What is the last successful IPv4 address that was reached?

10.10.1.17

Enter Ctrl+C to stop the trace.

Open configuration window

Click R1. Press ENTER and log in to the router.

Enter the show ip interface brief command to list the interfaces and their status. There are two IPv4 addresses on the router. One should have been recorded in Step 2a.

#### Question:

What is the other?

10.10.1.6 S0/0/1

Enter the show ip route command to list the networks to which the router is connected. Note that there are two networks connected to the Serial0/0/1 interface.

#### Question:

What are they?

10.10.1.4/30 S0/0/1

10.10.1.96/27 G0/1

Repeat steps 2e through 2g with R3 and record your answers.

10.10.1.17 (G0/1), 10.10.1.10 (S0/0/1)
10.10.1.8/30 (S0/0/1), 10.10.1.16/28 (G0/1)

Click R2. Press ENTER and log into the router.

Enter the show ip interface brief command and record your addresses.

10.10.1.2 S0/0/0, 10.10.1.9 S0/0/1, G0/0 & G0/1 unassigned/admin down

Run more tests if it helps visualize the problem. Simulation mode is available.

Close configuration window

### Propose a solution to solve the problem.

Compare your answers in Step 2 to the documentation you have available for the network.

#### Question:

What is the error?

Type your answers here.

What solution would you propose to correct the problem?

Type your answers here.

### Implement the plan.

Implement the solution you proposed in Step 3b.

### Verify that connectivity is restored.

From PC1 test connectivity to PC3.

From PC3 test connectivity to PC1.

#### Question:

Is the problem resolved?

Type your answers here.

### Document the solution.

## Test and Restore IPv6 Connectivity

### Use ipv6config and ping to verify connectivity.

Click PC2 and open the Command Prompt.

Enter the ipv6config /all command to collect the IPv6 information. Complete the Addressing Table with the IPv6 address, subnet prefix, and default gateway.

Click PC4 and open the Command Prompt.

Enter the ipv6config /all command to collect the IPv6 information. Complete the Addressing Table with the IPv6 address, subnet prefix, and default gateway.

Test connectivity between PC2 and PC4. The ping should fail.

### Locate the source of connectivity failure.

From PC2, enter the necessary command to trace the route to PC4.

#### Question:

What is the last successful IPv6 address that was reached?

2001:db8:1:3::2/64

The trace will eventually end after 30 attempts. Enter Ctrl+C to stop the trace before 30 attempts.

From PC4, enter the necessary command to trace the route to PC2.

#### Question:

What is the last successful IPv6 address that was reached?

None

Enter Ctrl+C to stop the trace.

Click R3. Press ENTER and log in to the router.

Enter the show ipv6 interface brief command to list the interfaces and their status. There are two IPv6 addresses on the router. One should match the gateway address recorded in Step 1d.

#### Question:

Is there a discrepancy?

Currently R2 S0/0/0 is show 10.10.1.2 in settings but it is 10.10.1.5 in the routing table

Run more tests if it helps visualize the problem. Simulation mode is available.

### Propose a solution to solve the problem.

Compare your answers in Step 2 to the documentation you have available for the network.

#### Question:

What is the error?

S0/0/0 on R2 is misconfigured.
IPv6 default gateway on PC4 is misconfigured.

What solution would you propose to correct the problem?

Update S0/0/0 configuration.
Updated IPv6 default gateway on PC4.

### Implement the plan.

Implement the solution you proposed in Step 3b.

### Verify that connectivity is restored.

From PC2 test connectivity to PC4.

From PC4 test connectivity to PC2.

#### Question:

Is the problem resolved? Yes

Type your answers here.

### Document the solution.

Noticed connectivity between PC1 & PC3 did not exist. Completed a ping and tracert. Ping failed and tracert from PC1 to PC3 showed that the last successful connection was 10.10.1.97, which is the IP address for R1 G0/1. From PC3 to PC1, the last successful connection was 10.10.1.17, which is the IP address for R3 G0/1. Completed a sh ip int br on both R1 & R3 and noticed their configurations were correct. Finally, conducted a  sh ip int br on R2 and noticed that the IP did not match the addressing table. Updated the IP address and mask. Tested the connection between PC1 & PC3 – it worked.

Noticed connectivity between PC2 & PC4 did not exist. Completed a ping and tracert. Ping failed and tracert from PC2 to PC4 showed that the last successful connection was 2001:db8:1:3::2, which is the IP address for R3 S0/0/1. From PC4 to PC2, did not have a successful connection. Completed a sh ipv6 int br on R1, R2 & R3 and noticed their configurations were correct. Since the connection from PC2 to PC4 stopped at R3, and PC4 didn’t get past R3, I verified the IP configuration of PC4. PC4 showed an IPv6 default gateway of fe80::2, which belongs to R2. PC4 default IPv6 gateway was updated to match R3 (fe80::3) Tested the connection between PC2 & PC4 – it worked.